Town of Mantua, Utah

PUBLIC WORKS CONSTRUCTION STANDARD DRAWINGS AND SPECIFICATIONS

REVISED AND ADOPTED NOVEMBER 2014

Hansen & Associates, Inc 538 North Main

Brigham City, UT 84302 Ph. 435-723-3491

Fax. 435-723-3492

##### TOWN OF MANTUA PUBLIC WORKS	 CONSTRUCTION STANDARD DRAWINGS AND SPECIFICATIONS

This document contains standard drawings depicting the adopted standards for designing and constructing public works facilities in the Town of Mantua, Utah. These standards also apply to the design and construction of privately owned and maintained roads set aside to serve new commercial and residential developments within the community boundaries.

Developments or projects which front along public roads owned and maintained by the State of Utah shall also comply with the applicable standards developed by the Utah Department of Transportation, except that Town of Mantua culinary water and sanitary sewer system facilities shall comply with Town of Mantua Public Works Construction Standards and Specifications.

##### Town of Mantua Public Works Construction Specifications

Unless otherwise specified herein on the standard drawings and specified by UDOT for roadway improvements along State Highways, construction of public works facilities and infrastructure in Mantua Town shall comply with the latest edition of the American Public Works Association (APWA) Manual of Standard Specifications which have been adopted be reference as a companion document to the standard drawings contained herein. Resolution of any inconsistencies between the included standard drawings and the adopted APWA Manual of Standard Specifications will be made by the Town Engineer. Developers, subdividers, and contractors doing work on public facilities or private development roads in Mantua Town shall be responsible for obtaining a copy of and becoming familiar with the referenced the latest edition of the APWA Manual of Standard Specifications.

Copies of the latest edition of APWA Manual of Standard Specifications can be obtained for $30.00 at the following location:

Utah LTAP Center Utah State University 8205 Old Main Hill

Logan, Utah 84322-8205

##### Standard Drawing Index	Sheet No.

Street Improvement Details - 80' Wide Major Collector Roads	1

Street Improvement Details - 66' Wide Minor Collector and Local Roads	2

Street Improvement Details - 66' Wide Collector and Major Local Roads	3

Street Drainage Details - Typical Dry Well Sump Installation	4

Typical Street Intersection - 66' Wide Minor Collector and Local Roads	5

Typical Cul-De-Sac Detail - For 66' Wide Local Roads	6

Typical Street Sign and Stop Sign & Post Details	7

Sanitary Sewer Details	8

Sanitary Sewer Manhole Details	9

Storm Drain Details	10

Culinary Water System Standard Details	11

Large Diameter Service Meter Details	12

Miscellaneous Culinary Water System Details	13

Standard Mailbox and Newspaper Box Details	14

| TOWN O F MANTUA PUBLIC W O R K S C ONSTR UCTION STANDAR D S Standard Drawing Index SIEET 1 - STREET IMPROVEMENT DETAILS - 80' WIDE MAJOR COLLECTOR ROAD SIEET 2 - STREET IMPROVEMENT DETALS - 66' WIDE MINOR COLLECTOR AND LOCAL ROADS SIEET 3 - STREET IMPROVEMENT DETALS - 66' COLLECTOR AND MAJOR LOCAL ROADS SIEET 4 - STREET DRAINAGE DETALS - TYPICAL DRY WELL SUMP INSTALLATION SIEET 5 - TYPICAL STREET INTERSECTION - 66' WIDE MINOR COLLECTOR AND LOCAL ROADS SIEET 6 - TYPICAL CUL;-OE-SAC DETAL - FOR 66' WIDE LOCAL ROADS SIEET 7 - TYPICAL STREET SIGN AND STOP SIGN & POST DETALS SIEET 8 - SANITARY SEWER DETALS SIEET 9 - SANITARY SEWER MANHOLE DETALS RECOMMENDED: SIEET 10 - STORM DRAIN DETALS SIEET tt - CULINARY WATER SYSTEM STANDARD DETALS SIEET 12 - LARGE DIAMETER SERVICE METER DETALS Jim Flint, P.E. Date SIEET 13 - MISCELLANEOUS CULINARY WATER SYSTEM DETALS SIEET 14 - STANDARD MALBOX AND NEWSPAPER BOX DETALS TOWN OF MANTUA PLANNING Date COMMISSION CHAIRMAN |  |  |
| --- | --- | --- |
| APPROVAL: | PUBLIC WORKS CONSTRUCTION SPECIF/CATIONS: | HANSE N & ASSO CIATES, INC. Consulting Engineers and Land Surveyors 538 Nort h Main Street, Brigham, Utah 84302 Visit us at w w w. haies. net Brigham City Ogden Logan (435) 723-3491 (801) 399-4905 (435) 752-8272 |
|  | WHERE APPLICABLE, AND UNLESS OTHERWISE SPECIFIED, CONSTRUCTION |  |
|  | OF PUBLIC WORKS FACILITIES AND INFRASTRUCTURE SHALL COMPLY WITH |  |
|  | THE 20 12 EDITION OF THE AMERICAN PUBLIC WORKS ASSOCIATION |  |
|  | MANUAL OF STANDARD SPECIFICATIONS PUBLISHED BY THE UTAH CHAPTER |  |
| MANTUA TOWN MAYOR Date | OF THE AMERICAN PUBLIC WORKS ASSOCIATION WHICH ARE ADOPTED BY |  |
|  | REFERENCE AS A COMPANION DOCUMENT TO THESE DRAWINGS. |  |
|  | RESOLUTION OF ANY INCONSISTENCIES BETWEEN THESE DRAWINGS AND |  |
|  | THE ADOPTED SPECIF/CATIONS WILL BE MADE BY THE TOWN ENGINEER. |  |
| MANTUA TOWN CLERK Date |  |  |

0

O_

0

f----

C)

## :s

0

N

n

CXJ 0

N

n

I

0

0

N

Ul

0

De

<(

0

z

<(

f---­

Ul

z

## :s

0

f----

<(

=i

z

f---­

<(

2

/ en u

L

0

u

C

0

+-'

Ul

C

s

0

f----

0

::J

+-' C 0

2

/ en u

L

0

u

C

0

+-'

Ul

u

/

z

| - . M£I£R PLAC£M£NT: - IN NEW CONSTRUCnON, INSTALL M£I£R AT C£NT£R OF LOT OR P£R AGENCY R£QUIR£M£NTS. 8. ALL M£r£RS AR£ TO 8£ INSTALLED IN TH£ PARK STRIP OR AS SHOWN £LSEWH£R£ IN THESE STANDARD DRAWINGS. C. DO NOT INSTALL M£r£RS UNDER DRN£WAY APPROACHES, SIDEWALKS, DR CURB &- GUTTER. - PIP£: INSTALL TYP£ 'K' COPPER PIP£ FOR 1.5" AND 2" M£I£R INSTALLAnONS FROM MAIN UN£ PIP£ TO PROPERTY UN£ COORDINATE WITH unuTY AGENCY FOR TYP£ OF PIP£ TO 8£ USED OUTS/0£ OF RIGHT-OF-WAY. - AL TERNATE: TURBIN£ M£I£RS AR£ R£QU/R£D ON ALL SYSTEMS USED £XCLUSN£LY FOR IRRIGAnON OR FIR£ PROTECnON. WHERE DOMESnc US£ IS APPUCABL£ US£ A STANDARD M£r£R. - BYPASS VALV£: LOCK IN OFF POSmON. - BLOCKING: US£ CLAY BRICK OR CONCR£I£ BLOCK. - CONCR£I£ BOX: - CENTER FRAM£ AND COVER OVER WAT£R M£I£R. 8. ALLOW 1 INCH CL£ARANC£ AROUND WATERUN£ WH£R£ LIN£ PASSES THROUGH WALL. SEAL OPENING WITH COMPRESS/BL£ SEAL 7. INSP£CnON: PRIOR TO BACKFILUNG AROUND TH£ M£r£R BOX, S£CUR£ INSP£CnON OF INSTALLAnON BY TOWN £NGIN££R. GRADE RING I Ya• CUSTOM SETT£R 5'x5'x4'd WITH BYPASS PRECAST BOX &- LID 2" CUSTOM SETT£R W/0 BOTTOM WITH BYPASS TEMPORARILY SET / PRECAST BOX AND POUR CONCRETE FOOnNG IN PLACE 4 • DRAIN GRAVEL BYPASS VALVE SEE NOTE 4 SECTION 2 " TURBO METER BRASS PIPE SEE NOTE 5 !!:: 0 Ii I SECTION A-A l ALTERNATE TURBO METER INSTALLATION SEE NOTE 3 1/2" AND 2" METER |  |  |  | -Y,"I (4}REO'D .. PIPE • • HOT DIP GALVANIZE .. - T£R FABRICATION ROMAC MJRG MECHANICAL JOINT RETAINER GLAND .. WALL PENETRATION DETAIL LADDER DETAIL NOT£ 1: OR OTHER METER BRAND P£R COMMUNITY WATER DEPARTMENT CONCRETE VAULT . 6x6x6.5 AMCOR PRECAST 6x6x6.5 AMCOR PRECAST <o CONCRETE VAULT I/ WALL PENETRAnON PIPE RESTRAINT --111-----'IP"'-='ll'-=>'11-4.-- -------./--•'11"·'-"'l -111--- 9 6.0' 6.0' PLAN PLAN TOP REINFORCED FO TOP REINFORCED FO HS-20 TRAFFIC HS-20 TRAmc LOADING LOADING - ANCHOR BOLTS *" ANCHOR BOLTS TYP. TYP. GROUT LINES IN GROUT LINES IN PRECAST OPENINGS PRECAST OPENINGS ALL FlmNGS SHALL BE ALL FlmNGS SHALL BE AWWA C-110 WITH 125 AWWA C-110 WITH 125 LB. FLANGES ALL PIPING LB. FLANGES ALL PIPING SHALL BE DUCnLE IRON SHALL BE DUCnLE IRON PIPE CLASS 350 PSI MIN. PIPE CLA.SS 350 PSI MIN. SECTION SECTION 3" WATER METER STATION 4" & 6" WATER METER STATION CONSTRUCnON R£0UIR£M£NTS: WHERE APPUCABL£ AND UNLESS OTH£RWIS£ SP£CIFl£0, CONSTRUCnON OF PUBUC WORKS FACIL/nES AND INFRASTRUCTURE SHALL COMPLY WITH THE LATEST £DmON OF THE AMERICAN PUBUC WORKS ASSOCIAnON MANUAL OF STANDARD SP£CIFICAnONS WHICH ARE ADOPTED BY REFERENCE AS A COMPANION DOCUMENT TO THESE DRAWINGS. RESOLUnON OF ANY INCONSISTENCIES BETWEEN THESE DRAWINGS AND TH£ ADOPTED SP£CIFICAnONS WILL 8£ MAD£ BY TH£ TOWN £NG/NEER. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | APPROVED MANTUA TOWN MAYOR |  | DATE | HANSEN & ASSOCIATES, INC. Consulting Engineers and Land Surveyors 538 North Moin Street, Brighom, Utoh 84302 Visit us at www.haies. net Brigham City Ogden Logan (435} 723-3491 (801} 399-4905 (435) 752-8272 | REVISION DATE: | Town of Mantua | SHEET: |
|  | JIM FLINT, P.E. FOR HANSEN AND ASSOCIATES | DATE |  |  |  |  | 9-30-14 t------------ DATE: 1/23/09 t------------ SCALE: NONE | Public Works Construction Standards | 12 OF 14 SHEETS |
|  |  |  |  |  |  |  |  | i LARGE DIAMETER SERVICE i METER DETAILS |  |

| PIP£ & FTTTlNG SCH£DUL£ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NO. | 0£SCRIPTION | TYP£ | J" UN£ | 4• UN£ | 6" UN£ |
| 7 | 5£NSUS W-JSODR M£T£R W/STl?AIN£R (S££ NOT£ 1) | Fl. | J" | - | - |
| 2 | 5£NSUS W-100DR M£T£R W/STRA1N£R (5££ NOT£ 1) | Fl. | - | 4 | - |
| J | 5£NSUS W-200DR M£T£R W/ STRA1N£R (S££ NOT£ 1) | Fl. | - | - | 6" |
|  | R£S/U£NT SE"AT G<T£ VAL W/IWIDWHffi. | Fl.xFI. | J" | - | - |
| 5 | R£S/U£NT SE"AT G<T£ VAL W/IWIDWHffi. | Fl.xFI. |  |  | 6" |
| 6 | ROMAC FCASO 1 Fl.ANG£D COUPUNG ADAPTcR | Fl.xP£ | J | 4 | 6 " |
| 7 | ROMAC DJ400 DISMANTLING JOINT | Fl.xFI. | J" | 4• | 6" |
| 8 | T££ | Fl. | J" |  | 6" |
| 9 | 90" £LBOW | Fl. | y | 4 | 6" |
| 10 | SPOOL Pl£C£ | Fl.xP£ | 3•x30" | 4•x30• | 6•x30• |
| 71 | SPOOL Pl£C£ | Fl.xFI. | J x,JI | 4 x29 | 6·x26• |
| 12 | SPOOL P/£C£ | Fl.xP£ | 3•x12• | 4"xt2 " | 6"x12" |
| 7J | SPOOL Pl£C£ | Fl.xP£ | :rxS6" | 4 xS6 | 6 "x54 1/2" |
| 14 | SPOOL Pl£C£ | Fl.xP£ | 3·xs6 | 4·x.s6 · | 6 "x54 1/2" |
| 15 | ROMAC 501 F1.£XIBL£ PIP£ COUPUNG | Fl.xP£ | 3'"x23" | 4"x26 1/2" | 6"x22 J/4 " |
| 16 | ROMAC WRG M£CIWI/CAL JOINT R£TAIN£R GWID |  | J" |  | 6" |
| 17 | ,J "x4 " REDUCER | Fl.xFI. | J | 4 | - |

Q_

Li_

0

O_

0

f-

(_')

### s

0

2

<(

I";

N

N

I";

I

0

0

N

(/)

0

u:'

<(

0

z

<(

f­

(/)

z

### s

0

f-

<(

=i

z

f­

<(

2

/

(J)

u

\.._

0

u

C

0

+--'

(/)

C

s

0

f-

0

::J

+--'

C

0

2

/

(J)

0

u

u

0

C

+--'

/

(/)

u z

|  | 3,000 PSI @ 28 DAYS CONCRETE BLOCKING PLAN VIEW 4" THICK x 2'-0" ;/ ' DIAMETER CONCRETE VALVE ' \ BOX STABILIZING PAD IN j,7. PAVED AND UNPAVED AREAS COMPACTED BACKFILLED EARTH FLANGE FtmNG 55 TAPPING SLEEVE WITH FULL GASKET. -FLANGE x MJ SEE NOTE 1 RESILIENT WEDGE VALVE RESTRAINT FLANGE SYSTEM REQUIRED IN HIGH PRESSURE AREAS CONCRETE BRICK BLOCKING SEE NOTE 2 POUR 3000 PSI CONCRETE BLOCKING AND FOOTING UNDER ASSEMBLY AS SHOWN SECTION A -A NOTES: - Concrete blocking is to be formed to ensure accesibility to fittings and poured against undisturbed earth. - Fittings ore to be completely wrapped with plastic, prior to pouring concrete. - Concrete ta be minimum 3,000 psi. @ 28 days. 4 " TO 12" STANDARD TAPPING SLEEVE AND VALVE ASSEMBLY DETAIL SCALE: NONE CONSTRUCnON R£0UIR£M£NTS: WH£R£ APPUCABL£, ANO UNLESS OTHERWISE SP£CIF/£0, CONSTRUCnON OF PUBUC WORKS FACILJn£S AND INFRASTRUCTURE SHALL COMPLY WITH TH£ LATEST £OmON OF TH£ AMERICAN PUBUC WORKS ASSOCIAnON MANUAL OF STANDARD SP£CIFICAnONS WHICH AR£ ADOPTED BY R£FER£NC£ AS A COMPANION DOCUMENT TO TH£S£ DRAWINGS. R£SOLUnON OF ANY INC0NSIST£NCl£S 8£TW££N TH£S£ DRAWINGS AND TH£ ADOPTED SP£CIFICAnONS WILL 8£ MAD£ BY TH£ TOWN £NGIN££R. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | JIM FLINT, P.E. DATE FOR HANSEN AND ASSOCIATES | APPROVED HANSEN & ASSOCIATES, INC. Consulting Engineers and Land Surveyors 538 North Moin Street. Brighom. Utah 84302 Visit us at www.haies.net Brigham City Ogden Logan (435} 723-3491 (801) 399-4905 {4J5) 752-8272 MANTUA TOWN MAYOR DATE | REVISION DATE: 9-30-14 1------------... DATE: 1 1/23/ 13 I-----------< SCALE: NONE | Town of Mantua Public Works Construction Standards | SHEET: 13 OF 14 SHEETS |
|  |  |  |  | MISCELLANEOUS CULINARY WATER SYSTEM DETAILS |  |

Li_

0

O_

0

f-

(_')

### s

0

2

<(

U)

I";

I

0

0

N

(/)

0

u:'

<(

0

z

<(

f­

(/)

z

### s

0

f-

<(

=i

z

f­

<(

2

/

(J)

u

\.._

0

u

C

0

+--'

(/)

C

s

0

f-

0

::J

+--'

C

0

2

/

(J)

u

0

u

C

0

+--'

(/)

/

u z

Li_

0

O_

0

f-

(_')

### s

0

2

<(

8"  TO   12"  FROM  FACE  OF CURB

30" WIDE CURB AND  GUTTER

# ==o

MAILBOX  PER USPS

I

NEWSPAPER  BOX

4 "x4" WOOD OR OTHER STYLE

/		POST PER  USPS WIDTH VARIES

SIDEWALK

3 '   6"  M!N.	I •	3'-6 "  MIN.	·1

CENTER  TO  CENTER  OF  POST

NEWSPAPER  BOX

FINISH GROUND

GENERAL NOTES:

- USE ANY COMMERCIALLY AVAILABLE MAILBOX MEETING U.S. POSTAL  SERVICE (USPS} REQUIREMENTS AND DETAILS ON THIS DRAWING.

- HEIGHTS TO MAILBOXES ARE SUPERCEDED BY USPS REQUIREMENTS IF  THEY  DIFFER FROM THOSE SHOWN HEREON.

- DO NOT EXTEND NEWSPAPER BOXES BEYOND THE FRONT OF THE MAILBOX WHEN THE MAILBOX  DOOR IS CLOSED.

- FOR MAILBOX AND NEWSPAPER STOP TURNOUT WIDTHS 8 FEET OR GREATER, REFER TO MAIL STOP TURNOUT DETAILED HEREON.

- IN THIS STANDARD DRAWING, DIFFERENT RESTRICTIONS ARE IMPOSED ON THE PERMITTED LOCATION OF DECORATIVE MAILBOX SUPPORTS OR HOLDERS, CONSTRUCTED OF MASONRY OR OTHER RIGID MASSES,  WHICH CAN SIGNIFICANTLY INCREASE THE HAZARD

I";

I

0

0

N

TYPICAL FOR SINGLE OR GROUP   MAILBOXES

DECORATIVE MASONRY OR OTHER DECORATIVE  TYPE  MAILBOX  SUPPORTS  ARE PERMITTED IN PARK STRIPS WHERE ROADWAY CURB AND GUTTER EXISTS, SO LONG AS THEY COMPLY  WITH  THE SETBACKS ABOVE AND  CONFORM  TO USPS REQUIREMENTS.

MAILBOX   IN   PARK  STRIP

NOT TO SCALE

# es

c::i

:5

o

FRONT VIEW

TYPICAL   GROUP  MAILBOX   AND   NEWSPAPER  BOX

NOT TO SCALE

MASONRY OR OTHER DECORATIVE TYPE

STRUCTURE COMPLYING WITH USPS	---------	-

POTENTIAL TO THE TRAVELING PUBLIC AND ROAD MAINTENANCE EQUIPMENT WHEN A LOW-YIELDING ROADSIDE OBSTRUCTION IS IMPACTED.

(/)

0

u:'

<(

0

z

MAILBOX  PER USPS

# ==o

i;i

ole.

t3

<(

f­

(/)

z

### s

0

f-

<(

=i

z

f­

<(

2

/

(J)

u

\.._

0

u

NEWSPAPER BOX 4"x4"   WOOD  POST REQ'

MIN. 4. 0' ALL WEATHER SHOULDER ASPHALT  PAVEMENT

2% SLOPE---

--------i ---,---.,...  E:l

ASPHALT  PAVEMENT

MIN. 8. 0' ALL WEATHER SHOULDER TURNOUT FOR DECORATIVE STYLE MAILBOX SUPPORTS

/

ole.

i.J..J

(.'.)

G	'-1-

EDGE OF TRAVELED WAY

i

MASONRY OR OTIICR	\ DECORATIVE TYPE MAILBOX REFERENCED IN NOTE NO.

4  AND  5 ABOVE.

C

0

+--'

(/)

C

s

0

f-

0

::J

+--'

C

0

TYPICAL FOR  SINGLE OR GROUP  MAILBOXES

MAILBOX ON RURAL OR LOCAL ROAD WITH ALL  WEATHER   SHOULDER

NOT TO SCALE

ROADSIDE   DRAINAGE   SWALE---­

MIN 12" DIA. DRAINAGE CULVERT WHERE REQ 'D TO FILL IN SWALE TO PROVIDE MAILBOX TURNOUT

o

Cl:l

""I;

a C'\J '-1-

MAIL  STOP   TURNOUT LAYOUT

NOT TO SCALE

2

/

(J)

u

\.._

0

DECORATIVE   MAILBOX   ON  RURAL   OR  LOCAL   ROAD   WITH  ALL   WEATHER  SHOULDER

NOT TO SCALE

u

C

0

+--'

APPROVED

HANSEN & ASSOCIATES,  INC.

REVISION  DATE:

9-30-14

#### Town   of   Mantua

•tso·	··ro

Public  Works   Construction  Standards

SHEET:

(/)

Consulting Engineers and Land Surveyors

s-r	AR'"S

538  North  Moin  Street,  Brighom,  Utoh  84302  >----    -------<	••A	V   A

&1e1u    n A nen   80' V 	14

u	JIM FLINT, P.E.	DATE

/	FOR HANSEN AND ASSOCIATES

z

MANTUA TOWN  MAYOR

DATE

Visit   us   at   www.haies.net	DATE·   5/ 19/ 2014	lffl11l	:A     IYl	1w,;;; n.;,r11r,;;;n	:A

Brigham City	Ogden	Logan	.	A &lft

(4.15)  123-3491      rso1J  399-4905	(4.15)  752-8272	I  111'1Ul1    a.,,

SCALE:  AS    NOTED   FOR  COLLECTOR  AND  LOCAL   COMMUNITY  ROADS

OF  14  SHEETS